ISO9001:2015 Quality System Certification

Committed To Creating The Best

Intelligent Control Terminal

Contents

File information	3

Change records	3

Change review	3

1. Contact Us	4

2. Company Environment	4

3. Product Description	5

3.1 Product Models	5

3.2 Products Features	7

3.3 Products Size	8

3.4 Products Parameters	9

4. Hardware Introduction	11

4.1 Definition of terminal pin	11

5. Reliability Test	13

5.1 ESD Test	14

5.2 High low Temperature Aging Test	15

5.3 Electrical Fast Transient test	16

5.4 Lightning Surge Test	18

6. Software Download	20

7. Description PC Configuration Software	20

7.1 Develop software	20

7.2 Protocol Configuration	24

8. Development Of The Document	25

File information

| Writer |  | The date of writing |  |
| --- | --- | --- | --- |
| Reviewer |  | The date of the review |  |
| Approver |  | The date of approval |  |

# Change records

| date | author | version | Change instructions |
| --- | --- | --- | --- |

# Change review

| date | Reviewer | opinion |
| --- | --- | --- |

- Contact Us

Shenzhen Kinseal Intelligent Control Technology Co., LTD

Floor 3, Yaoji Building 1,  Xinhetongfufu Industrial Zone,

Fuhai Street, Bao 'an District, Shenzhen

Company Web Site：www.kinseal.com

Business  Of  Selling：086-136 4144 2131

Technical Support：086-186 8246 9902

- Company Environment

- Product Description

  - Product Models

| 型号 | 规格说明 |
| --- | --- |
| AMZ101P24W01RES | AMZ series, 10.1 inch,Serial screen,10-30V,serial port 1: RS232, serial 128Mbyte SPI Nand Flash, Resistive touch screen |
| AMZ101P24W01RDS | AMZ series, 10.1 inch,Serial screen,10-30V,serial port 1: RS485, serial 128Mbyte SPI Nand Flash, Resistive touch screen |
| AMZ101P24W01REE | AMZ series, 10.1 inch,Serial screen,10-30V,serial port 1: RS232, serial port 2: RS232,128Mbyte SPI Nand Flash, Resistive touch screen |
| AMZ101P24W01RED | AMZ series, 10.1 inch,Serial screen,10-30V,serial port 1: RS485 serial port 2: RS485,128Mbyte SPI Nand Flash, No touch |

Model Definitions

Figure Shows：

AMZ 101 P24 W01 R E S

E indicates that serial port 2 is RS232，D is RS485，S is Not supported

E indicates that serial port 1 is 232，D indicates that serial port 1 is 485

R means resistance touch, C means capacitive touch, N means no touch

Indicates the flash type，01：SPI NAND Flash；02：SPI NOR Flash

Represents a 10.1-inch bare screen without a housing

Indicates the product model size 10.1 inches

Represents  AMZ series HMI

## 3.2 Products Features

| Power Voltage | DC10-30V |
| --- | --- |
| Backlight Adjustment | Support |
| Touch | 4-wire resistive touch screen，Capacitive touch screen support |
| LCD resolution | 1024X600 |
| USB1 | Micro USB |
| USB2 | TYPE-A |
| LCD | TN TFT LCD，250cd/m2 |
| COM1 communication | RS232/RS485communication(2 choose 1) |
| COM2 communication | RS232/RS485通讯（2 choose 1） |
| Storage | 128Mbyte SPI NAND FLASH |
| Power Protection | To prevent the reverse，Distinguish the positive and negative poles of power supply |
| RTC | Support |
| Save power failure data | Support |
| Download | SD card，USB download，U disk |

## 3.3 Products Size

| Display Size | Product Model | Overall Dimension | Display Visual Area | Panel Cutout | Overall Thickness |
| --- | --- | --- | --- | --- | --- |
| 10.1 in. | AMZ101P24 | 257.7*148.3mm | 222.7*125.3mm | 247.5*120.4mm | 22mm |

## 3.4 Products Parameters

| Product Specification |  |  |
| --- | --- | --- |
| Hardware Performance | Module | AMZ101P24 |
|  | Display Size | 10.1" TFT LCD |
|  | Resolution | 1024X600 |
|  | Colors | 260000 |
|  | Brightness | 250 cd/m2 |
|  | Backlight | LED |
|  | LED life | 20000 hours |
|  | Touch screen | 4-wire resistive touch screen touch screen（Surface hardness 4H）Capacitive touch is supported |
|  | CPU | 32-bit 600MHz ARM9 built-in 32MB DDR2 memory |
|  | Memory | 128Mbyte SPI Nand Flash |
|  | RTC | Real-time clock built-in |
|  | Ethernet | Unsupported |
|  | Save power failure data | Support |
|  | USB | A USB2.0 Device port，A USB2.0 HOST port |
|  | Program download Mode | USB，U disk，SD card |
|  | U disk | Support |
|  | Communication port | COM1：RS232,RS485(2 choose 1) COM2：RS232,RS485(2 choose 1) |
| Electrical Specifications | Maximum power consumption | 3.5W |
|  | The rated voltage | DC 10-30V |
|  | Power protection | +/-2KV lightning surge protection capability |
|  | The allowed loss of power | <5ms |
|  | CE&ROHS | Comply with EN61000-6-2:2005, EN61000-6-4:2007 standards;Lightning surge +/ -2kV, EFT: +/ -4kV;Electrostatic contact discharge +/-8KV;Electrostatic air discharge +/-10KV. |
| Environmental Specifications | Operating temperature | -20℃~50℃ |
|  | Storage temperature | -40℃~70℃ |
|  | Environmental humidity | 10~90%RH(non-condensing) |
|  | Vibration | 10-25Hz (X, Y, Z direction, 2g/30 min) |
|  | Cooling way | Natural air cooling |
| The Other Parameters | Protection grade | The front panel conforms to IP65 (with flat plate cabinet installation), and the rear panel conforms to IP20 |
|  | Display Active Area | 222.7*125.3mm |
|  | Panel Cutout | 247.5*120.4mm |
|  | Overall dimensions | 257.7*148.3mm |
|  | The weight | 280g |

- Hardware Introduction

  - Definition of terminal pin

| Port Definitions |  |
| --- | --- |
| Device Position Number | Descriptions |
|  | DC24V power interface and COM1 communication input interface |
|  | COM2 communication interface |
|  | COM1 RS485 communication module interface, plug 485 module |
|  | COM1 RS232 communication module interface, plug 232 module |
|  | R3, R4 resistance, when COM1 selects TTL, a 0 euro resistor is required |
|  | COM2 RS485 or RS232 communication module interface |
|  | TYPE A USB download port, default USB download port, through the software system settings can be switched to USB drive mode |
|  | Micro USB |
|  | SD card interface |

| COM1The output interface——① |  |  |  |
| --- | --- | --- | --- |
| Pin | Definition | Descriptions |  |
| 1 | GND | Negative power input |  |
| 2 | RXD | RS232_RX,RS485B,TTL_TXD | 3 choose 1 |
| 3 | TXD | RS232_TX,RS485A,TTL_TXD |  |
| 4 | blank | blank |  |
| 5 | +24V | Positive input of power supply |  |

| COM2The output interface——② |  |  |  |
| --- | --- | --- | --- |
| Pin | Definition | Descriptions |  |
| 1 | RXD | RS232_RX,RS485B | 2 choose 1 |
| 2 | TXD | RS232_TX,RS485A |  |
| 3 | GND | GND |  |

| COM1Communication Mode-③④⑤status selection |  |  |
| --- | --- | --- |
| COM1 communication methods / device location | ③④---COM1 module | ⑤---resistance R3 R4 |
| RS232 | Plug in the RS232 module | blank |
| RS485 | Plug in the RS485 module | blank |
| TTL | blank | Attach a 0-degree resistance |

| COM2Communication Mode-⑥status selection |  |
| --- | --- |
| COM2 communication methods / device location | ⑥---com2 module |
| RS232 | Plug in the RS232 module |
| RS485 | Plug in the RS485 module |

- Reliability Test

All products of  company  have carried out a series of  reliability tests before mass production , including ESD test, high and low temperature aging test, EFT and so on, to ensure the quality of  products

## 5.1 ESD Test

Execution standard：IEC 61000-4-2

Test process: Lay the product flat on the test table, and conduct contact and air discharge in turn around the touch screen iron buckle and the display area, as showing above. Check the screen is weather reset and displayed abnormally.

Testing Data

| Product Models | Discharge type | Discharge value | Test Results |
| --- | --- | --- | --- |
| AMZ101P24W01RES | Contact | +/-8KV； | No reboot, system halt or Abnormal display. Function is normal |
|  | Air | +/-10KV； | No reboot, system halt or Abnormal display. Function is normal |
| AMZ101P24W01RDS | Contact | +/-8KV； | No reboot, system halt or Abnormal display. Function is normal |
|  | Air | +/-10KV； | No reboot, system halt or Abnormal display. Function is normal |
| AMZ101P24W01REE | Contact | +/-8KV； | No reboot, system halt or Abnormal display. Function is normal |
|  | Air | +/-10KV； | No reboot, system halt or Abnormal display. Function is normal |
| AMZ101P24W01RED | Contact | +/-8KV； | No reboot, system halt or Abnormal display. Function is normal |
|  | Air | +/-10KV； | No reboot, system halt or Abnormal display. Function is normal |

## 5.2 High low Temperature Aging Test

Test Environment: high and low temperature aging test box

Test Temperature: -20° to 70°

Test Process: Place the product in the high and low temperature aging test box. Through 70° high temperature, -20° low temperature, high and low temperature alternating transformation aging test. Observing  the product  whether resets ,restarts  and shows abnormal display, abnormal function, and other phenomena during the test process.

The Test Data

| Product model | The temperature | Humidity | The test results |
| --- | --- | --- | --- |
| AMZ101P24 | The high temperature of 70 ° | 60% | No reboot, system halt or Abnormal display. Function is normal |
|  | Low temperature - 20 ° | 60% | No reboot, system halt or Abnormal display. Function is normal |
|  | Alternating high and low temperature (-20°~70°) | 60% | No reboot, system halt or Abnormal display. Function is normal |

## 5.3 Electrical Fast Transient test

Implementation Standard :IEC 61000-4-4

Test Process: Lay the product flat on the test bench and supply power to the screen through the pulse group generator, coupling pulse group. The diagram above. Check the product whether resets, restarts, and shows  abnormal display on the screen.

The Testing Data

| Product Models | Test Standard | Test Port | The Test Results |
| --- | --- | --- | --- |
| AMZ101P24W01RES | EFT +/-4KV; | The power port | There is no flicker under 1KV , and flicker above 1KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |
|  |  | Communication port | There is no flicker under 2.5KV , and flicker above 2.5KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |
| AMZ101P24W01RDS | EFT +/-4KV; | The power port | There is no flicker under 1KV , and flicker above 1KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |
|  |  | Communication port | There is no flicker under 2.5KV , and flicker above 2.5KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |
| AMZ101P24W01REE | EFT +/-4KV; | The power port | There is no flicker under 1KV , and flicker above 1KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |
|  |  | Communication port | There is no flicker under 2.5KV , and flicker above 2.5KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |
| AMZ101P24W01RED | EFT +/-4KV; | The power port | There is no flicker under 1KV , and flicker above 1KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |
|  |  | Communication port | There is no flicker under 2.5KV , and flicker above 2.5KV on the screen, but there is no reboot, system halt or abnormal display. Function is normal. |

## 5.4 Lightning Surge Test

The Test Data

| Product Models | Test Standard | Impedance | The Test Results |
| --- | --- | --- | --- |
| AMZ101P24W01RES | COUPL | 12Ω | 2KV screen no flickering, 2KV screen has flashing, but no restart, crash, flower screen and other anomalies. Function is OK |
| AMZ101P24W01RDS | COUPL | 12Ω | 2KV screen no flickering, 2KV screen has flashing, but no restart, crash, flower screen and other anomalies. Function is OK |
| AMZ101P24W01REE | COUPL | 12Ω | 2KV screen no flickering, 2KV screen has flashing, but no restart, crash, flower screen and other anomalies. Function is OK |
| AMZ101P24W01RED | COUPL | 12Ω | 2KV screen no flickering, 2KV screen has flashing, but no restart, crash, flower screen and other anomalies. Function is OK |

- Software Download

Products configuration software and driver installation method, please go to the official website of Kinseal Intelligent Control company download center to download the following files:《Kinseal_setup_v3.1 configuration software》、《KinsealStudio software installation manual》、《KinsealStudio software driver installation manual》。

Notes: For Windows 7, the system will automatically install the driver after installing KinSealStudio software. For Windows XP and Windows 8, the driver installation fails due to operating system limitations，According to the first《Manual for Disabling digital signatures for Windows 8 and 10》Disabling digital Signatures，then according to the《KinsealStudio software driver installation manual》Manually install the driver.

Software Download Address：http://www.kinseal.com/list-4-1.html

# 7. Description PC Configuration Software

## 7.1 Develop software

### PC Software KinSealStudio is a tool that users can edit arbitrarily. All application programs are developed based on it. KinSealStudio composes rich controls, which can be arbitrarily combined to achieve the functions that users want：

### Features Includings：

| Switch Button | Including "Bit Button" "Word Button" "Indicator" "Screen Button" "Function Button" "Polymorphic Button" touch the connected device to monitor the status |
| --- | --- |
| Numerical Input and Display | Including a variety of base input and display, ASCII code input and display, used to display the monitoring address value;Another time display is used to display the real-time time |
| Flow Block | An animated graphic that simulates the flow of liquid in a pipe |
| Static text/table/scale | A variety of basic shapes, including lines, circles, ellipses, rectangles, etc |
| Image Display and GIF animation | A picture display box that displays one or more pictures |
| Instrument | Bar chart, meter, ring, showing a value of some state of data |
| Message Display | Displays messages that be edited by user |
| Alarm Display | Display the alarm information of the current device (divided into digital alarm and analog alarm), Alarm settings must be configured before using the control |
| Dynamic Alarm | Used to display the current alarm, which is different from the alarm control is the dynamic alarm bar in the form of text scrolling display the current alarm content |
| X Y Curve | Real time and dynamic display of data trend of data collector |
| History Curve | Displays the data saved by the history collector in curvilinear form |
| Recipe | Create a recipe like menu |
| Multifunction Button | A switch button, through the button can be very convenient to achieve a variety of functional requirements |
| Trend Chart | Draw reference curves of multiple data to accurately and intuitively judge the change trend of a certain value in a period of time |
| Dropdownlist | Used to select the corresponding state function |
| Data group | To display the data change from register address N to register address N as a curve |
| Motion trail | Control address data by dragging a scroll block |
| Qr code | Dynamic generation of QR code, through scanning into the url, payment and other functions |

Extended Function Of Host Configuration Software：

| Macro Instruction | C programming language, to achieve a variety of more complex logic or functions |
| --- | --- |
| PLC Control | HMI is controlled by PLC |
| Language | Support for multiple languages |
| Data Collection | Data acquisition for temperature, pressure, humidity, etc |
| Data Transmission | Refers to the transmission of data over the same type of address. The transmission mode can be periodic (how many seconds) or triggered |
| HMI Protection | It is set that HMI can normally use HMI within a certain period of time. If the time exceeds the time specified by the user, HMI will jump to the specified screen previously set by the user. In the specified screen, the user only places the "Panel protection unlock button" under the function button. |
| File Protection | Whether to enter a password when opening the project |
| User password Level | Set the user permission and password. Enter the password to access the corresponding permission |
| Boot Screen | Users can customize the startup Logo screen |
| Online Simulation | Online simulation enables you to communicate with relevant devices such as PLC through personal computer (HMI configuration software needs to be installed first) without HMI |
| Offline Simulation | Before the picture is compiled and downloaded to the HMI device, the offline simulation function of HMIStudio can be used to check the correctness of the configuration picture and the effect display |
| Supports Multiple Controller Communication Protocols | Suitable for a variety of PLC, inverter, servo controller, SCM control system (Mitsubishi, Panasonic, Omron, Delta, Letter Jie, Fatek, Siemens, Keens, LG, and other major PLC) user operation only need to choose to call directly on the software |
| Custom Add Gallery | Support custom add gallery, users can according to the need to intercept their favorite picture loaded into the custom gallery to call |
| Keyboard | Support Chinese and English keyboard input, users can freely switch to use |
| Gallery | Image library rich, support Png, Jpg, Gif, Bmp and other formats of pictures, vector image library, arbitrary zoom without aliasing |
| Baud Rate Range | Support serial baud rate range of 1200-230400 |

## 7.2 Protocol Configuration

Users can run MODBUS RTU, Mitsubishi, Siemens, Delta,Xinje and other protocols through the configuration of the upper computer.

Open KinSealStudio configuration software and click on new project to select the desired communication protocol in new project. You can also modify the communicate on protocol you need in the project  by opening the protocol under COM1 port Settings. The diagram below.

# 8. Development Of The Document

In order to complete the products development more faster, beginners can log in the official website http://www.kinseal.com，enter the "Download center" bar, download the corresponding "communication case", "project case", "video tutorial" and other technical documents. For more technical knowledge, please call: 086-13641442131 or e-mail kinseal@kinseal.com

Committed To Creating The Best

Intelligent Control Terminal